Avaliação da qualidade da água em dois setores de Ouro Preto

A visão de “Gênero” refere-se aos diferentes papéis, direitos e responsabilidades de mulheres e homens e as relações entre eles. As mulheres desempenham o mais importante papel dentro da gestão da água em suas residências e podem contribuir para melhoria da qualidade de vida, da saúde e da promoção da cidadania.  O município de Ouro Preto possui uma estação de tratamento de água e grande parte da população recebe apenas água clorada cuja qualidade não é monitorada. O objetivo do trabalho foi verificar a gestão da água pelas donas de casa em dois setores de Ouro Preto, Setor A e Setor B, com população sócio - econômica heterogênea.  Foram realizadas avaliação da qualidade físico-química e bacteriológica da água nas residências amostradas e avaliação do perfil sócio-econômico nas residências sorteadas, através de questionário próprio e da avaliação da ficha A da Secretaria da Saúde de Ouro Preto.  Nas amostras coletadas nos dois setores não foram encontradas bactérias coliformes termotolerantes; os coliformes totais apareceram em 13,21% das amostras do setor A e 4,18% no setor B. O pH da água apresentou variações no setor A (10% pH de 4 a 5,  55% pH 6,8 e 35% pH de 7,2 a 7,8), no setor B todas as amostras apresentaram pH 6,8. A concentração de cloro nas amostras no setor A foram Cl= 1,5 4%, Cl=1 20%, Cl=0,5 62%, Cl=0,5 14%  no setor B foram: Cl=1 6%, Cl=0,5 83%, Cl=0,5 11% . A frequencia de falta de água para o setor A foi 53% semanalmente, 10% mensalmente e 37% não falta;  para o setor B 6% mensalmente e 94% não falta. Tanto no setor A quanto no setor B, 94%, as pessoas bebem água filtrada, porém no A, 6% não bebem água filtrada e, no setor B 6% bebem água mineral. No setor A, 90% apresentaram as caixas d`água tampadas adequadamente e no Setor B 100%.  A renda familiar no setor A apresentou as seguintes variações em R$ - 12% de 175,00 a 350,00; 29% de 350,01 a 700,00; 30% de 700,00 a 1.050,00; 27% de 1.050,01 a 1.750,00 e 2% de 1.750,00 a 3.500,00. No setor B a renda familiar apresentou as seguintes variações: 29% de 350,01 a 700,00; 6% de 700,00 a 1.050,00; 47% de 1.750,00 a 3.500,00 e 6% de 7.000,00 a 10.550,00. O grau de escolaridade apresentou as seguintes variações, setor A- 72% ensino fundamental incompleto, 4% ensino fundamental completo, 4% ensino médio incompleto, 4% ensino médio completo 16% e ensino técnico 4%; no setor B as variações foram as seguintes- 12% ensino fundamental incompleto, 23% ensino fundamental completo, 6% ensino médio incompleto, 23% ensino médio, 18% ensino superior incompleto, 12% ensino superior, 6% pós graduação. Os resultados mostraram que é necessário um programa de monitoramento da água pela secretaria de abastecimento de água de Ouro Preto: não há controle físico químico e microbiológico no sistema de abastecimento de água nos dois setores, e que as mulheres do setor A necessitam de capacitação para o mercado de trabalho.

Drinking water quality in two sectors of Ouro Preto – MG -Brazil

The vision of "Gender" refers to different roles, rights and responsibilities of men and women and relations between them. Women play a more important role in the water management in their homes and can contribute to improving quality of life, health and the promotion of citizenship. The city of Ouro Preto has a water treatment plant and much of the population receives only chlorinated water whose quality is not monitored. The aim of this study was to investigate the water management by housewives in two sectors of Ouro Preto, A Sector and B Sector whose population is heterogeneous socially and economically. We evaluated the physicochemical and bacteriological qualities water in the sampled households, and we also checked their socio-economic profile through questionnaires. In the samples collected in the two sectors were not found fecal coliform bacteria, total coliforms appeared in 13,21% of samples from A Sector and 4,18% sector B. In the A sector the water pH varied: 10% pH from 4 to 5, 55% pH= 6.8, and 35% pH from7,2 to 7,8 ; in B sector, all samples showed pH= 6,8. The chlorine concentration in the samples in the A sector was: 4% Cl=1.5, 20% Cl=1, 62% C =0.5, 14% Cl=0.5; in B sector the chlorine concentration was: 6%Cl=1, 83% Cl = 0.5, 11% Cl = 0.5.   The frequency of water shortage in A sector was 53% weekly, 10% monthly and 37% of houses had no lack of water; for sector B, the frequency of water shortage is 6% monthly and 94% did not have lack water. In both sectors 94% people drink filtered water, but in A sector 6% do not drink filtered water and, in B sector 6% drink bottled water. In A sector, 90% of homes had the water tank capped properly in Sector B 100%. Family income in A sector made the following changes  12% from R$175,00 to R$350,00, 29% from R$350,01 to R$700,00, 30% from R$700,00 to R$1050,00; 27% R$1050,01 to R$1750,00 and 2% from R$1750,00 to R$3500,00. In sector B family income showed the following variations: 29% from R$350,01 to R$700,00, 6% from R$700,00 to R$1050,00; 47% from R$1750,00 to R$3500,00 and R$7000,00 6% to R$10.550,00. The level of education showed the following variations - A sector: 72% incomplete elementary education, 4% complete elementary education, 4% incomplete high school 16% high school and 16% high school technical education. In B sector the changes were the following-12% incomplete elementary education, 23% complete elementary education, 6% incomplete high school, 23% high school, 18% incomplete higher education, 12% higher education, 6% PhD (Philosophic Doctor) graduate. The results showed that there is need for water monitoring program by the Department of the water supply of Ouro Preto because there is no physicochemical and microbiological control at system of water supply in both sectors. Although women in A sector have a higher educational level than B sector, both sectors need to acquire skills for labor market. Ouvir

Ler foneticamente

- Dicionário - Ver dicionário detalhado

- preposição

  - at the

- pronome

  - we

  - us